World Directory of

Cold and Ultra-cold Neutron Sources

compiled by Klaus H. Gobrecht

Version 4 - 2007

## Contents

page							page

Contents	2

Preface	3

Argonne IPNS	5

Austin, Texas	6

Beijing  CARR	7

Berlin HMI	8

Bloomington LENS	9

Budapest KFKI	10

Cairo_ETRR2	11

Chalk River	12

Chilton ISIS	13

Daejeon HANARO	14

Delft HOR	15

Dubna JINR	16

Gaithersburg NIST	17

Garching FRM2	18

Garching UCN and VCN	19

Gatchina PNPI	20

Geesthacht GKSS	21

Grenoble ILL	22

Jülich FZ	23

Kjeller_JEEP2	24

Kyoto_RRI	25

Los Alamos	26

Los Alamos UCN	27

Lucas Heights_OPAL	28

Lungtan	29

Mainz TRIGA	30

Mianyang	31

Oak Ridge HFIR	32

Oak Ridge SNS	33

Raleigh NCSU	34

Saclay CEA	35

Serpong	36

Tokai-mura JRR	37

Tokai-mura JSNS	38

Tsukuba KEK	39

Vienna (Wien)	40

Villigen SINQ	41

Villigen SUNS	42

Theory and Computation Groups	43

Bariloche	43

Florence	43

Stuttgart	43

Organisations, Meetings	44

ACoM	44

ECNS, ICNS, and ACNS	45

IAEA	46

ICANS	47

IGORR	48

People	49

Glossary	50

note for word 2000+ users: by clicking on the site name in the content list you arrive
                                          directly at the site page.

## Preface

The first time, to my knowledge, that an effort was made to collect all possible information about the many cold neutron sources (CNS) in the world, was by Guy Gistau (then L'Air Liquide) at the IGORR6 meeting in Taejon (now Daejeon), Korea. Guy’s findings, however, have never been really divulged.

Having been involved in the gestation and birth of quite a number of CNS myself, I decided to compile, from both my own files and Guy Gistau's data base, a first version of the "World Directory of CNS".

In view of the growing activity in the field of ultra-cold neutrons (UCN), I have renamed the “World Directory of CNS” (from version 3) to:

"World Directory of Cold and Ultra-cold Neutron Sources".

The CNS are presented in alphabetical order of the sites.

As of today (2007), I have identified 36 operating CNS, 6 under construction and about 6 planned. Some of the planned CNS are mentioned at sites which are already in the list for other reasons, like ISIS Second Target Station at Chilton, PIK at Gatchina and IBR-2M at Dubna.

The following sites that had used a CNS have since been shut down (with the year of shut down):

Grenoble (Siloette)	1970					Argonne (ZING-P)	1975
Saclay (EL3)		1977					Argonne (ZING-P')	1980
Karlsruhe		1981					Helsinki		1984 (?)
Aldermaston		1988					Harwell		1990
Beijing (HWRR)	1995 (?)				Brookhaven		1999
Risoe			2000					Garching (FRM)	2000
Cornell 		2001					Juelich		2005

Dedicated existing or planned sources of UCN have been included in the site list below.  Most have been developed until now essentially at places which also exploit a classical cold source.  However, in future this may no longer be true (e.g. Mainz, CERN, PSI, LANL).

The last pages contain a Glossary of used and useful terms, and a list of people mentioned in this document.

To access useful links (site information, proceedings and transactions of conferences, workshops, meetings), click on the underlined blue text.

On the site www.ottosix.com, you can also find an extended bibliography on CNS and UCN sources.

The following site also lists some interesting neutron sources (not CNS!):

http://neutron.neutron-eu.net/n_about/n_where/worldwide.

I wish to thank all former colleagues and all others, who contributed and helped me with their comments to write and edit this directory.

Klaus H. Gobrecht, HYERES, France, (e-mail: klaus@ottosix.com),         June 2007

## Argonne IPNS

Organisation:	Argonne National Laboratory

Installation:	IPNS (since 1981)

Type: 	spallation source, 450 MeV, 30 Hz, beam power 8 kW, 
target: depleted uranium

Address:	9700 South Cass Ave
Argonne, IL 60439, USA

Home page:	http://www.anl.gov

Contact: 	John ("Jack") M. Carpenter

Phone / fax:	+1 630-252-5519 ,	 fax: -4163

e-mail:	jmcarpenter@anl.gov

Secretary: 	Diane Hoffmann   –6485

CNS:		one liquid CH4 ("F"),      two solid CH4  ("C" and "H")

temperature:		110 K ("F") / 25 K ("C" and "H")	power: 	50 W each

total power available: 	300 W 				volume: 	0.5 L each

heat removal:		direct He cooling ("C" and "H"), circulating ("F")

remarks: 	"C" and "H" contain ~10v% Al foam, are decoupled and fixed,
"C" is grooved; "H" is poisoned;
"F" is decoupled, and poisoned

----------------------------------------------------------------------------------------------------

projects:	IPNS studies cold moderators for SNS (see site Oak Ridge SNS in this document)

recent publications: ICANS-XVI (Neuss DE 2003): on irradiated H2 (proc. p.707), on liquid and solid CH4 (proc. pp. 719 and 783) 
ICANS-XVIII (China 2007): Bradlley J. Micklich reports about "Metal hydrides as moderators for pulsed spallation neutron sources", Id18.
Also at ICANSXVIII : : Prospects for a Very Cold Neutron Source (on a long pulse linear accelerator), by Bradley J. Micklich and John M. Carpenter, Id17

see also: Workshop on "Applications of a Very Cold Neutron Source" at ANL, Aug.'05
              Workshop on "Present Status and Future of Very Cold Neutron Applications" 
              at PSI Villigen, Switzerland Feb. '06

A nice introduction to neutron moderation by Jack Carpenter: www.neutron.anl.gov/NeutronProduction.pdf

## Austin, Texas

Organisation:	The University of Texas at Austin

Installation:	Nuclear Engineering Teaching Laboratory, NETL

Type: 	TRIGA Mark II reactor, LEU, 1.1 MW since 1992, up to 1.5 GW in pulsed mode

Address:	J.J.Pickle Research Campus, Building 159
Austin, TX 78712, USA

Home page:	http://www.me.utexas.edu

Contact: 	Donna J. O’Kelly

Phone / fax:	+1 512-232-4174 ,	 fax: -471-4589

e-mail:	djokelly@mail.utexas.edu

CNS:		"Texas Cold Neutron Source" TCNS

moderator: 			mesitylene		pressure:		~ 0 kPa

temperature:		40 K 			cooling power needs: 	16 W

(12 W ambient)

total power available: 	22 W 			moderator volume: 		 88 ml

heat removal:		liquid neon thermal siphon

remarks: 		new cryo-refrigerator in 2004

----------------------------------------------------------------------------------------------------

projects:	PGNAA

recent publications:  K. Ünlü, C. Rios-Martinez, and B. W. Wehring, 
"The University of Texas Cold Neutron Source"
Nucl. Instr. and Meth. in Phys. Res. A353, 397 (1994).

Donna J. O’Kelly "INIE AT UT"
Proc. of IGORR10, NIST 2005

UCN: This TRIGA reactor could be a perfect candidate for the installation of a UCN
          source

## Beijing  CARR

Organisation:	China Institute of Atomic Energy (CIAE)

Installation:	CARR

Type: 	pool reactor (60 MW LEU), Øth=8·1014 n/cm²s, under construction

Address:	PO box 275(33), Beijing 102413, China,

Home page:	http://www.ciae.ac.cn/

Contact: 	Prof. Yuan Luzheng

Phone / fax:	+86 10-69358140, 	fax: -69357008

e-mail:	yuanluzheng@iris.ciae.ac.cn

Co-workers:	Dr. Feng Shen (e-mail: shenf@iris.ciae.ac.cn)

Dr. Quing Feng Yu (qingfengyu@hotmail.com)

---------------------------

CNS Project :

moderator: 	H2			phase:		2-phase liquid/gas

volume: 	~1 L			temperature: 	23 K

pressure:	24 kPa		heat removal:	gravity (single tube) *)

power:	800 W			refrigerator:		?

material:	Al 6061
___________________

*) + direct cooling

to be built in collaboration with the Cryogenic Laboratory Xi'an Jiao Tong University, start end 2007.

---------------------------

remarks: 	The actual reactor HWRR has operated a CNS from 1988 to 1990

recent publications:

"CARR-CNS with Crescent-shape Moderator Cell and Sub-cooling Helium Jacket around Cell" by Qingfeng Yu, Quanke Feng, Takeshi Kawai, Feng Shen, Luzheng Yuan, Liang Cheng at the Symposium in Commemoration of the 10th Anniversary of HANARO in Daejeon, Korea, April 2005.  http://hanaro2005.kaeri.re.kr/data/proceeding/CN-O-13.pdf.

"Development of CNS in CARR" by Quanke Feng and Feng Shen at the 5th UCN/CNS Workshop, St. Petersburg 2005:

The Chinese Academy of Sciences plans a spallation neutron source with cold source (project CSNS), cf ICANS17_Id70, 2007: "Neutronics study of the coupled para-hydrogen moderator for CSNS" by Wen Yin, Tianjiao Liang, and Qiwei Yan.

## Berlin HMI

Organisation:	Hahn-Meitner-Institut / BENSC

Installation:	BER-2

Type: 	pool reactor,10 MW, LEU, Be-reflector, critical in 1991, unperturbed Øth= 3·1014 n/cm²s,

Address:	Glienicker Str. 100
14109 Berlin, Germany

Home page:	http://www.hmi.de

Contact: 	Dr. Herbert Krohn

Phone / fax:	+49-30 8062–2740		fax: -2999

e-mail:	krohn@hmi.de

Secretary: 	Antje Thomas –2742

CNS:	one, operating since 1991

moderator: 	H2			phase:		super-critical

volume: 	1 L			temperature: 	27 – 30 K

pressure:	1.3 / 1.9 MPa 	heat removal:	forced (circulator)

power:	1800 W		refrigerator:		AL, screw,

material: chamber:	AlMg3			vacuum thimble:	AlMg3

performance:	90 % reliability, if not stopped during reactor shut-down

----------------------------------------------------------------------------------------------------

remarks:	reactor does not need to shut down in case of CNS failure
(room temperature cooling loop).

recent publications: CNS gain curve in Neutron News 3/93 p.17

                                nothing more recent ?

## Bloomington LENS

Organisation:	Indiana University Cyclotron Facility

installation:	LENS (Low Energy Neutron Source)

type: 	accelerator based neutron source, target Be, 
H+ 50 mA, >11 MeV (upgraded)

address:	IUCF, 2401 Milo B. Simpson Ln, Bloomington, In 47408

home page:	http://www.iucf.indiana.edu

contact: 	David V. Baxter

phone / fax:	+1-812-855-8337	fax:	-5533

e-mail:	baxterd@indiana.edu

co-workers:		Mike_Snow (phone: -7914), Mark_Leuschner (phone: 812-856-1721)

--------------------------------------------------------------------

Cold source: Installation on the PL-7 proton beam line (operation 2005)

(variable pulse width up to 1.5 ms) :

Cold moderator	solid CH4 at 22K or less.

UCN:		for testing new moderators (e.g. solid_O2 by Chris Lavell in the 6th UCN/CN Workshop in Russia in July 2007)

recent publ.:	proc. UCN-CNS_4th_Workshop (PNPI 2003) by M. Leuschner

"LENS: a new pulsed neutron source for research and education" by M. Leuschner et al. in J. Res. Natl. Inst. Stand. Technol. 110, 153-155 (2005)]

"LENS produces its first “cold” neutron beam" by Mark Leuschner
April 26, 2005, see www.iucf.indiana.edu/news/news.php?story=24

ICANSXVIII_Id181, 2007
"Slow Neutron Sources: Cold, Very Cold, and Ultracold"
by W. M. Snow.

## Budapest KFKI

Organisation:	AEKI

Installation:		WWR at KFKI (new operation licence until 2013)

Type: 	pool reactor 10 MW, LEU 39%, Øth=2.5·1014 n/cm²s,

Address:	KFKI, P.O.Box 49
H-1525 Budapest, Hungary

Home page:	http://www.kfki.hu/brr/reactdpt.html

Contact: 	Sándor Tözsér

Phone / fax:	+36 1 395-9139, fax:  -9162

e-mail:	tozser@sunserv.kfki.hu

Co-workers:	Laszlo Rosta (rosta@szfki.hu), fax +36 1 392-2501
István Vidovszky (vidov@sunserv.kfki.hu)

CNS:	operating since Feb. 2001

moderator: 	H2			temperature:	20 K, pressure:	150 kPa

volume: 	0.5 L			phase:		sub-cooled liquid

power:	250 W 		heat removal:	directly cooled

material: chamber:	AlMg5			vacuum thimble:	AlMg5

refrigerator:	Linde, Brayton cycle, 1 turbine, 2 screw-compr. 18+37 kW(el.)

----------------------------------------------------------------------------------------------------

remarks:	reactor does not need to shut down in case of CNS failure
(room temperature cooling loop).
CNS has been built in collaboration with PNPI Gatchina.

There are plans for refurbishment (HEU => LEU + replacing the actual CNS by a more performing one within the next five years).

recent publications: ICNS 2001 (Munich), proceedings in Appl. Phys. A (Suppl.1), 2002 (L. Rosta et al.)

IGORR9 to 11 (I. Vidovszky)

"Performance and operation of LH2 CNS at the Budapest Research Reactor" by 
T. Grosz at the 6th_UCN/CN-Workshop in Russia in July 2007.

## Cairo_ETRR2

Organisation:	ATOMIC ENERGY AUTHORITY OF EGYPT

Installation:	ETRR-2, built by INVAP, Argentina, still not commissioned

Type: 	pool reactor 22 MW, Øth=2.7·1014 n/cm²s, since 1997

Address:	INSHAS Nuclear Research Center, 13759 CAIRO, Egypt

Home page:	http://www.etrr2-aea.org.eg/

Contact: 	Dr. Mohammed K. Shaat (shaat@etrr2-aea.org.eg)

Phone / fax:	+2-02-4691753	fax -1754

e-mail:	m_shaat30@hotmail.com

Other contacts:

reactor state:	not operating due to nuclear safety issues

-------------------------------------------------------------

CNS: 			call for tender Oct.1999, no official news since

moderator: 		probably LH2 ,

--------------------------

most recent serious publication:	Neutron news Vol. 11, No. 2, 2000

more news ?

## Chalk River

Organisation:	Atomic Energy of Canada Ltd (AECL) 
and National Research Council (NRC)

Installation:		NRU (first operation in 1957, continuing until 2012)

Type:			120 MW, D2O coolant and moderator, Øth=4·1014 n/cm²s

Home page:		http://www.nrureactor.ca/

Contact: 		Ian_P._Swainson

Secretary:		Niki_Schrie

Address:		Chalk River Laboratories, Chalk River Ontario, K0J 1J0, Canada

Phone/fax:		(613) 584 8293 or -3995, 	fax:	(613) 584 4040

Co-workers at NRC:	John_Root   (613) 584 8297
Eric_C._Svensson,

project team at AECL:	Albert Garland Lee (leeA@aecl.ca)
Raguy M. Rabbat (rabbatr@aecl.ca), head of project group CNS
Denis Seehoye (seehoyed@aecl.ca)

----------------------------------------

CNS:		none existing, but at least one CNS is proposed in the new Canadian Neutron Facility (CNF), a joint AECL/NRC project, which will be based on a new reactor as replacement of the NRU. Funding problems actually postpone the government decision.

More news ?

## Chilton ISIS

Organisation:	Council for the Central Laboratory of the Research Councils

Installation:	ISIS Facility

Type: 	spallation source (800 MeV, 0.2 mA, 50 Hz)

Address:	Rutherford Appleton Laboratory

Chilton, Didcot, Oxon OX11 0QX, UK

Home page:	http://www.isis.rl.ac.uk/

Contact: 	Tim Broome

Phone / fax:	+44 1235 44 6255, fax: +44 1235 44 5607

e-mail:	t.a.broome@rl.ac.uk

CNS:	two (hydrogen and methane), since 1985

Moderator: 	H2 / CH4		volume: 	0.5/0.8 litre

temperature:	22 / 100 K	 	phase:	liquid

pressure:	1 / 0.4 MPa		power:	300 W each

refrigerator	Sulzer / Philips 	cycle:		Brayton / Stirling

heat removal:	forced convection (circulator)

performance:	the hydrogen source works with a 25 Hz beam.

----------------------------------------------------------------------------------------------------

remarks:	Second Target Station design and construction is in progress, incorporating two cold moderators.

recent publications: 	ECNS'99 in Budapest: EPL3 (A.D. Taylor),

Proc. AcoM-V Workshop at HMI (March 2001)
published in J. of Neutron Research

ICNS 2001 (Munich): proceedings in Appl. Phys. A (Suppl.1), 2002 (T.A. Broome)

ICANSXVIII: "Re-design of the ISIS first target station for improved neutronics" 
by S. Ansell - Id111, 2007

ICANSXVIII: "Second coupled moderator for the ISIS second target station" 
by S. Ansell - Id110, 2007
New design with composite moderator substance (H2/CH4).

## Daejeon HANARO

Organisation:	KAERI

Installation:	HANARO

Type: 	open pool reactor, 30 MW (LEU), Øth=5·1014 n/cm²s, 
D2O reflector

Address:	P.O. Box 105, Yuson, 
Daejeon, 305-600 Korea

Home page:	http://hanaro.kaeri.re.kr/

Contact: 	Kye Hong Lee

Phone / fax:	+82 42 868-2277, fax:-8610

e-mail:	khlee@kaeri.re.kr

Co-workers: 		Jungwoon Choi (jwchoi@kaeri.re.kr)

CNS:		under construction (operational in 2008)

Moderator: 		H2 ,       phase:	liquid, 		temperature:	20 K

volume: 		1.3 L			pressure:		160 kPa

material: chamber:		Al6061 T6		vacuum thimble:	Zircaloy

heat removal:		gravity, two-phase thermal siphon

power:		about 1000 W

----------------------------------------------------------------------------------------------------

Remarks:		the CNS Vertical hole and beam tube are already built
	new shielding plug (design MTF) with 5 neutron guides

recent publications		Proc. IGORR8 (Munich 2001): C.O. Choi et al. p.103

Proc. ICEC19 (Grenoble 2002): C.O. Choi et al. p.123

"HANARO Cold Neutron Research Facility Project" by Y.-J. Kim in
Proc._UCN-CNS_4th_Workshop (PNPI 2003)

Proc. IGORR10 (NIST 2005): Y.J. Yu, K.H. Lee, H.R. Kim
"Current Status of the HANARO CNS"

Proc. IGORR2007 : "Measurement of Void Fraction in Hydrogen Moderator Used for Moderator Cell of HANARO Cold Neutron Source" by Myong-Seop Kim, 
Jungwoon Choi, et al.

## Delft HOR

Organisation:	TU-Delft, Interfaculty Reactor Institute

Installation:		HOR

Type: 	pool reactor 2 MW, operation since 1963, Øth=0.25·1014 n/cm²s,

Address:	Mekelweg 15
2629 JB Delft, The Nederlands

Home page:	Reactor Institute Delft

Contact: 	Prof. Adrian Verkooijen

Phone / fax:	+31 15 278-6614,	 fax:	+31 15 278-8430

e-mail:	a.h.m.verkooijen@iri.tudelft.nl

CNS:	in the design phase

Moderator: 	H2			phase:		liquid

heat removal:	directly cooled ?	pressure:		??

----------------------------------------------------------------------------------------------------

remarks:	CNS to be built in collaboration with PNPI Gatchina in the frame of a HOR upgrade

recent publications: IGORR8 (Munich 2001), IGORR9 (Sydney 2003) 
			"Options for the Delft Advanced Neutron Source" 
                                  by A.H.M. VERKOOIJEN, & al.

## Dubna JINR

Organisation:	Joint Institute for Nuclear Research (JINR), 
Frank Lab. of Neutron Physics (FLNP)

Installation:	IBR-2, fuel: PuO2, peak unperturbed Øth=270·1014 n/cm²s,

Type: 	pulsed reactor since 1977, average power 2 MW

Address:	141980 Dubna, Russia

Home page:	http://nfdfn.jinr.ru/flnph/

Contact: 	A.A.Beliakov, E.P.Shabalin

Phone / fax:	+7-09621-65657 or -65253, 	fax -65085 or -65882

e-mail:	beliakov@nf.jinr.ru

CNS:	one, operational since 1999

Moderator: 	CH4			temperature:		30 - 70 K

volume: 	1.2 L		 	phase:		solid

specific power:	100 mW/g		working pressure:	0.6 MPa

heat removal:	directly cooled

performance: 	gain factor = 20

----------------------------------------------------------------------------------------------------

Remarks:	strong radiolysis effects

A modernization program has been started: IBR-2M with three CNS (operational in 2010). E.P.Shabalin studies cold moderators for IBR-2M *).

recent publications: ACoM IV in 2/99, and ICANS-XV (Tsukuba 2000): 21.5

ICANS18_Id147: "Complex of moderators for the IBR-2M reactor" 
                               by V. Ananiev, A. Belyakov, & al.

ICANS18_Id178: "Experimental Study of Swelling of Irradiated Solid Methane 
                              during Annealing" by E. Shabalin, & al.

ICANS18_Id213: "The experimental and calculated density of vibrational states and UCN loss coefficients of perflu" by Y. Pokotilovski, & al. *)

*) see also 6th_UCN/CN Workshop in Russia in July 2007

## Gaithersburg NIST

Organisation:	National Institute of Standards and Technology

Installation:	NBSR, operational since 1967

Type: 	pool reactor, 20 MW, HEU, D2O reflector, Øth=4·1014 n/cm²s,

Address:	100 Bureau Drive
Gaithersburg, MD 20899, USA

Home page:		http://rrdjazz.nist.gov

Contact: 	Dr. Robert ("Bob") E. Williams

Phone / fax:	+1 301-975-6876, 	fax: +1 301-921-9847

e-mail:	robert.williams@nist.gov

CNS:	operating since 1995

moderator: 	H2			temperature:		20 K

effective volume: 	5 L 			phase:		liquid

pressure:	100 kPa		power:		1200 W

material: chamber:	Al 6061		vacuum thimble:	Al 6061

refrigerator:	CVI (Columbus OHIO),1 screw,

heat removal:	gravity (thermal siphon)

performance:	see http://www.ncnr.nist.gov/coldgains/ (April 2002)

--------------------------------------------------------------------

remark:	reactor does not need to shut down in case of CNS failure
(room temperature cooling loop).

recent publications: Proc. IGORR9 (Sydney 2003): SNS-04 (session 4)

Symposium in Commemoration of the 10th Anniversary of HANARO -

Daejeon, Korea, April 2005: "Issues in the Design of a Cold Neutron Source"
by Mike Rowe (now retired).

---------------------------------------------------------------------

UCN group around M. Scott Dewey and Geoffrey Greene, P.R. Huffman

On a cold neutron beam: Superfluid 4He, cooled below 1K, magnetic confinement,

Home page: http://www.itl.nist.gov/div898/pubs/ar/ar2000/node18.html

recent publication:	"Measurement of the neutron lifetime by counting trapped 
protons in a cold neutron beam" by M. Scott Dewey at the 5th UCN/CN Workshop in Peterhof (RU) 2005

See also "Progress towards Precision Measurement of the Neutron Lifetime using Magnetically Trapped Ultracold Neutrons" by L. Yang at same workshop 2005.

## Garching FRM2

Organisation:	Technische Universität München

Installation:	FRM-II, started operation in 03/2003

Type: 	pool reactor, 20 MW compact core (HEU), Øth=8·1014 n/cm²s, D2O-reflector

Address:	Reaktorstation
85748 Garching, Germany

Home page:	http://www.frm2.tu-muenchen.de/

Contact: 	Dr. Ingo Neuhaus

Phone / fax:	+49 89 289 1-2183, 	fax:	-2191

e-mail:	ingo.neuhaus@frm2.tum.de

Secretary: 	  -2154

CNS:	one, operational since 2003

moderator: 	D2 + 5% H2 		temperature:		25 K

eff. volume: 	15 L 			phase:		liquid

pressure:	150 kPa		power:		5000 W

material: chamber:	Al 6061		vacuum thimble:	Zircaloy

refrigerator	Linde, 2 turbines, 1 screw compressor,	500 kW(el.)

heat removal:	gravitational (single tube thermal siphon)

remarks:	N2 liner, low pressure metal hydride storage
reactor has to stop in case of CNS failure

----------------------------------------------------------------------------------------------------

remark: The vertical VCN/UCN guide is not in use.

recent publications:	ICANS-XV (Tsukuba 2000): 21.8 and 23.18, 
Proc. IGORR8 (Munich 2001): E. Gutsmiedl et al. p.175, 297.

IGORR10 (at NIST): "The FRM-II Hot and Cold Neutron Source" 
by Chr. Müller, E. Gutsmiedl, A. Röhrmoser, A. Scheuer, & al.

Symposium in Commemoration of the 10th Anniversary of HANARO 
Daejeon, Korea, April 2005: "Commissioning of the Cold Source at the FRM-II"
by E. Gutsmiedl, &al.

"Special Constructional Design Features of Cold and Hot Neutron Source and Confirmation/Verification during Commissioning of the Hot and Cold Neutron Source at the FRM-II" by E. Gutsmiedl, C. Müller, D. Päthe, and A. Scheuer. at IGORR10 (NIST 2005)

## Garching UCN and VCN

UCN group around Prof. Stephan Paul (stephan.paul@physik.tu-muenchen.de):

Project "mini-D2": Solid deuterium cooled to He-temperature, now (2006) no longer on a cold neutron beam position close to the CNS, but in a through-going beam tube (SR6), contact:	Dr. Erwin Gutsmiedl (erwin.gutsmiedl@frm2.tum.de)

co-workers: Igor Altarev, Joachim Hartmann, Wolfgang Schott, et al.

Home page:	http://www.e18.physik.tu-muenchen.de/research/ucn/minid2/

recent publications: ECNS2007 (Lund): "Production and Physics with Ultra Cold Neutrons" by Axel Müller, Igor Altarev, Erwin Gutsmiedl, 
Joachim Hartmann, & al.

"PENeLOPE and AbEx - towards a precise neutron lifetime measurement" 
by I. Altarev, & al. at the 5th_UCN/CN Workshop in Peterhof (RU) 2005.

"Constructional design of the Ultra cold Neutron source of the FRM II"
by A. Scheuer et.al. at the 5th_UCN/CN Workshop in Peterhof (RU) 2005.

"The very cold beam on MIRA at the Forschungsneutronenquelle Heinz Maier-Leibnitz (FRM II)" by R. Georgii (private communication, bibliography on ottosix site).

"Fundamental Neutron Physics at the Eastern Neutron Guide Hall of the FRM-II" by I. Altarev, A. Frei, E. Gutsmiedl, F.J. Hartmann, & al., 2006 (private communication, bibliography on ottosix site).

"PENeLOPE: On the way towards a precise neutron lifetime measurement" 
by R. Picker at the 6th_UCN/CN Workshop in Russia 2007.

## Gatchina PNPI

Organisation:	PNPI (Russian Academy of Sciences)

Installation:	WWM-R (operation license until 2015)

Type: 	pool reactor 14 to 16 MW, Be-reflector, Øth=0.8·1014 n/cm²s,

Address:	Gatchina, Leningrad District, 188350, Russia

Home page:	http://www.pnpi.spb.ru/

Contact: 	Anatoli Serebrov

Phone and fax:	+7 812 71 300 72

e-mail:	serebrov@pnpi.spb.ru

Co-workers:	Victor Mityukhlyaev (vicmit@pnpi.spb.ru)
Arcady Zakharov (arcady@pnpi.spb.ru)

CNS:	one vertical since 1986, not operating these days
one horizontal in a beam tube for testing cold moderators

moderator: 	D2+ X% H2 		temperature:		20 K

volume: 	0.5 L			phase:	sub-cooled liquid

power:	4000 W, used for either the vertical or the horizontal CNS

refrigerator	Brayton cycle, 2 turbines, piston compressors

heat removal:	gravity thermal siphon + direct cooling

__________________________________________________________________

Remarks:	vertical source was used for polarized VCN and UCN production.

There is also a horizontal solid D2 source.

There are plans for a new CNS + guide hall

recent publications: A.P. Serebrov and I.S. Altarev in Journal of Neutron Research Vol.1, N°4, more information in *).

PIK reactor (100 MW, Øth=3·1014 n/cm²s) under construction with at least one CNS
see proceedings IGORR9 (Sydney 2003)

--------------------------

UCN: 	Solid deuterium cooled to below 10 K *)

PNPI Gatchina organizes a series of international UCN/CN Workshops, 6 already *).

See also the page forthcoming events at the ottosix website.

other publications: UCN Workshop Los Alamos 2000 (see Los_Alamos)

*) see PNPI_UCN home page

## Geesthacht GKSS

Organisation:	GKSS Forschungszentrum

Installation:	FRG2

Type: 	MTR - pool reactor, 5 MW, HEU, compact, Øth= 1.3·1014 n/cm²s

Address:	GKSS, Postfach 1160,
21495 Geesthacht, Germany

Home page	http://www.gkss.de/

Contact: 	Dr. Wolfgang Knop

Phone / fax:	+49 41 52 87-1234 		fax: -1338

e-mail:	knop@gkss.de

CNS:	one, operating since 1988

moderator: 	H2			temperature:		25 - 35 K

volume: 	0.8 L			phase:		super-critical

pressure	1.3 - 1.7 MPa	power:		1355 W

material: chamber:	AlMg3			vacuum thimble:	AlMg3

heat removal:	forced (cold circulator), can work with natural convection

refrigerator	AL, 1 turbine, 1 screw compressor	160 kW(el.)

----------------------------------------------------------------------------------------------------

remarks:	reactor does not need to shut down in case of CNS failure
(room temperature cooling loop).

In 2007: Installation of a new moderator chamber (hemispherical instead of lenticular). 
60 % increase of CN output is expected, new cooling power demand should be 1625 W

----------------------------------------------------------------------------------------------------

recent publications:	"New Moderator Chamber of the FRG-1 Cold Neutron Source
for the Increase of Cold Neutron Flux" by P. Schreiner, W. Knop at IGORR2007 (Lyon 2007)
Same authors at ECNS4 in Lund (SE) 2007

## Grenoble ILL

Organisation:	Institut Laue-Langevin (ILL)

Installation:	HFR

Type: 	pool reactor 57 MW, HEU, D2O reflector, Øth=15·1014 n/cm²s,

Address:	6 rue Jules Horowitz, BP 156
38042 Grenoble Cedex 9, France

Home page:	http://www.ill.fr

Head of reactor:	Hervé Guyon

Contact: 	Jean Marie Astruc

Phone / fax:	+33 476-207220, fax -961143

Secretary: 	–207237

CNS:	two, one vertical (SFV3), operating since 1985,
one horizontal (SFH), operating since 1987

moderator: 	D2			temperature:		25 K

volume: 	27 L (SFV3),	6 L (SFH) 	phase:	liquid

material: chamber:	Al99%			vacuum thimble:	Zircaloy

power:	6500 W (SFV3),	3000 W (SFH)

refrigerator	AL, 2 turbines, 1 screw compressor	625 kWel.

heat removal:	gravity, thermal siphon (SFV3),	bubble pump (SFH)

----------------------------------------------------------------------------------------------------

remarks:	reactor has to stop in case of CNS failure. 
One vertical guide in SFV3 for VCN and UCN production.
SFV1 was operating from 1972 to 1985.

A third CNS is planned (H. Guyon, private communication)

----------------------------------------------------------------------------------------------------

recent publications: K. Gobrecht in Proc. Workshop on Cold Sources at LANSCE, Los Alamos, 1990, p.19
H. Guyon at various workshops, also at IGORR2007, session 1.

UCN Group around Peter Geltenbort

Steyerl turbine + 3 UCN guides

recent publications:	ICANS-XV (Tsukuba 2000): 22.2,
V.V. Nesvizhevsky et al. in Nature 415, 297 (2002),

Several contributions at the 6th_UCN/CN Workshop in July 2007 in Russia.

## Jülich FZ

Organisation:	Forschungszentrum (FZ) Jülich (former KFA)

Installation:	reactor FRJ-2  now definitely shut down

Type: 	DIDO 23MW, D2O reflector, Øth=3·1014 n/cm²s

Address:	52425 Jülich, Germany

Home page:	http://www.jcns.info/about_jcns/

Contact: 	Dr. Gunter Damm (Johannes Wolters retired)

Phone / fax:	+49 2461 61-3011	fax:	-3841

e-mail:	g.damm@fz-juelich.de

Status:	the reactor was definitely shut down in May 2006

CNS:	one, operating since 1971

moderator: 	H2		phase:		sub-cooled liquid

pressure:	250 kPa	temperature: 	19 K

volume: 	1 L		heat transfer:	gravity (thermal siphon)

power:	700 W 	refrigerator:	Linde

remark: 	reactor does not need to shut down in case of CNS failure
(room temperature cooling loop).

----------------------------------------------------------------------------------------------------

New project:

ESS    (Jan. 2003 : realization postponed sine die)
European Spallation Source, project group hosted by FZ Jülich (home page: http://www.fz-juelich.de/ess/), contact Dr. Günter Bauer..........but:

JESSICA:		Target/Reflector Mock-up and Cold Moderator Test Facility

"Solid Cold Moderators for High Power Neutron Sources" by G. Bauer at the 5th UCN/CN Workshop in Peterhof (RU) 2005.

contact:		H. Conrad, H. Stelzer (cold moderators)

phone: +49 (0)2461 61 3680,	Fax: +49 (0)2461 61 6816

recent publications:	Proc. ACoM Workshop in Juelich (Sept. 2002)

ICANS-XVI (Neuss 2003)

11th Int. Conf. on Nuclear Engineering in Tokyo, JAPAN, 04/03 (ICONE11-36422):

"JESSICA, THE TEST FACILITY FOR AN ADVANCED COLD MODERATOR SYSTEM FOR ESS" by H. Stelzer, & al.

## Kjeller_JEEP2

Organisation:	INSTITUTT FOR ENERGITEKNIKK

Installation:	Jeep-2

Type: 	pool reactor 2 MW, D2O-reflector, Øth=0.2·1014 n/cm²s,

Address:	P.O. BOX 40, 2007 Kjeller, Norway

Home page:	http://www.ife.no/

Contact: 	K. H. Bendiksen,

Phone/fax:	+47-63-806000, -806275,	fax:	-816356

e-mail:	kjell.bendiksen@ife.no

Co-worker:	Kenneth D. Knudsen  (e-mail: knudsen@ife.no)

Phone/fax:	+47-63-806084, 		fax:	-810920

CNS:	one, operating since 1975

moderator: 	H2		phase:		liquid

volume: 	0.5 L		power:		60 W

refrigerator:	Philips	heat transfer: 	gravity (thermal siphon)

temperature:	21 K		pressure:		150 KPa

----------------------------------------------------------------------------------------------------

Remarks:	reactor does not need to shut down in case of CNS failure

recent publications: Neutron News Vol. 11, No. 4, 2000

## Kyoto_RRI

Organisation:	Research Reactor Institute, Kyoto University

Installation:	KUR, shut down in 2005 for 2 years ?

Type: 	pool reactor 5 MW (HEU), Øth=0.3·1014 n/cm²s,
may reopen with only 1 MW power

Address:	Kumatori-cho, Sennan-gun, 
Osaka, 590-0494 Japan

Home page:	http://www.rri.kyoto-u.ac.jp

Contact: 	Takeshi_Kawai moved to China

Phone / fax:	+81-724-51-2371, 	fax:+81-724-51-2600

other contacts : 	Yuji Kawabata (kawabata@rri.kyoto-u.ac.jp)
Fumihiro Yoshino (yoshino@rri.kyoto-u.ac.jp)

CNS:	one, operating from 1987 to 2005

moderator: 	D2 		phase:		liquid

pressure:	200 kPa	temperature:		25 K

volume: 	4 L		power:		240 W (100 W nucl.)

heat removal:	gravity (thermal siphon)

refrigerator	Sulzer + Nippon Sanso Co.

----------------------------------------------------------------------------------------------------

recent publications: ICANS-XV (Tsukuba 2000): 23.19 (T. Kawai  et. al.),
                                IGORR9 (Sydney 2003) session 4.

"Cold neutron production in solid and liquid CH4 moderators" by N. Morishima and T. Mitsuyasu in Nucl.Inst&Meth. in Physics A, Volume 517,1-3, 2004, 295-300.

"Cold neutron production in solid and liquid CH4 moderators. II: on the reentrant-hole configuration" by T. Mitsuyasu, N. Morishima, & al. in Nucl.Inst&Meth. in Physics A, Volume_537,3, 2005, 610-613.

--------------------------------------------------------------------------------

_

UCN activity:

N. Morishima et al.: superfluid 4He (ICNS_2001 (Munich): poster C-2)

VCN Workshop in PSI Feb. 13-14, 2006:

"Very Cold Neutron Radiography for High Contrast Imaging" by Yuji Kawabata

## Los Alamos  LANSCE

Organisation:	Los Alamos National Laboratory

Installation:	LANSCE

Type: 	spallation source

Address:	Manuel Lujan Jr. Neutron Scattering Center, Mail Stop H805
Los Alamos, NM 87545, USA

Home page:	http://lansce.lanl.gov/

Contact: 	Gary J. Russell 

Phone / fax:	1-505 667 2937,	fax: 1-505 665 2676

Secretary: 	1-505 667-5005

CNS:	one, operating since 1985

moderator: 	H2		phase:		supercritical

temperature:	20 K		pressure:		1.4 MPa

volume: 	1 L		power:		160 W

heat removal:	forced (cold circulator)

refrigerator:	PSI (Koch), piston expander, screw, 185 kW(el.)

----------------------------------------------------------------------------------------------------

recent publications: ICNS 2001 (Munich): poster C-7 (A.J. Hurd).
Proc. AcoM Workshop at HMI (March 2001) 
(to be published in J. of Neutron Research).

## Los Alamos UCN

LANL UCN group at P25, MS H846:

Solid deuterium cooled to He-temperature (in collaboration with Caltech)

home page:	http://www.lanl.gov/projects/UCN2000/  (a new one under construction)
contact: 	Alexander Saunders  
Tel: +1 505 665 3090	Fax +1 505 665 7920

or:	 Thomas J. Bowles	(tjb@lanl.gov)
 phone:	+1 505 667-3937 		Fax: +1 505 665-4121
recent publication: UCN_3 Workshop 2001 in Pushkin (see Gatchina)

recent overview publication:

"Fundamental Physics with Cold Neutrons" by 
C.D. Bass, & al. at LANL 2006 (see ottosix website)

FP12 has the highest pulsed cold neutron intensity for nuclear physics in the world (http://p23.lanl.gov/len/npdg/).

"New Pulsed Cold Neutron Beam Line for Fundamental Nuclear Physics at LANSCE"
by P.-N. Seo, J. D. Bowman, & al. in J.Res Natl.Inst.Stand.Technol. 110 (2005)145.

ICANS18_Id44
"Scoping study for an electric dipole moment beam line at the Materials Test Station"
by G._Muhrer and Eric Pitcher

"The NPDGamma Experiment" by W. Scott Wilburn at the 5th_UCN/CN Workshop in Peterhof (RU) 2005, also by R. Mahurin at the 6th_UCN/CN Workshop.

## Lucas Heights_OPAL

Organisation:	ANSTO (Australian Nuclear Science & Technology Organisation)

Installation:	OPAL

Type:                  pool reactor, (20 MW, LEU, D2O reflector, Øth=5·1014 n/cm²s)

Address:	ANSTO, New Illawarra Rd, Lucas Heights 
PMB 1, Menai NSW, 2234, Australia

Home page:	http://www.ansto.gov.au

Contact: 	Dr. Ross Miller

Phone / fax:	+61 2 9717-3334, 	fax:	-3606

e-mail:	rmx@ansto.gov.au

Co-worker:	Dr. Robert A. (Rob) Robinson, phone -9204,	 fax -3606 
e-mail: 	rro@ansto.gov.au

CNS:	one, developed in collaboration with PNPI Gatchina

moderator: 	D2 			temperature:		20 K

eff. volume: 	15 L 			phase:		subcooled liquid

pressure:	150 kPa		power:		4000 W

material: chamber:	AlMg5			vacuum thimble:	Zircaloy

heat removal:	gravity (thermal siphon) and direct cooling

refrigerator	AL, 1 turbine,   2 screw compressors,	2 X 250 kW(el.)

remark: 	reactor will not need to shut down in case of CNS failure
(room temperature cooling loop).

--------------------------------------------------------------------------------------------------

- recent publications:	 IGORR9 (Sydney 2003), Session 4 (V. Mityukhlyaev)
IGORR10 (NIST 2005) 
IGORR2007 (LYON) "The OPAL Reactor" by R. Miller and T. Irwin (ANSTO), and J.P. Ordoñez (INVAP).

"Design of OPAL CNS, for ANSTO, Australia" by N. Masriera et O. Lovotti (INVAP) at the 5th UCN/CN Workshop in Peterhof (RU) 2005.

"General Approach to Safety and Protection of a Cold Neutron 
Source" by N. Masriera (INVAP) at the 5th UCN/CN Workshop in Peterhof (RU) 2005.

## Lungtan

Organisation:	Institute of Nuclear Energy Research (INER)

Installation:	TRR-2, refurbishing cancelled in 2002

Type: 	pool reactor, (goal: 20 MW, Øth=2·1014 n/cm²s)

Address:	1000, Wenhua Road, Chiaan Village,
Lungtan, Taoyuan, Taiwan, 325 Republic of China

Home page:	www.iner.gov.tw

Contact: 	Tsun-Neng Yang

Phone / fax:	+886 3 4711-400-		fax:	-408

e-mail:	tnyang@iner.gov.tw

Co-worker:	Yu-Hao Huang, C.H. Lee

CNS:		 (goal: LH2, 3.6 l, 3kW, thermal siphon)

recent publ.: 		Proc. ICANS-XV (Tsukuba 2000): 21.1, and 23.19

Proc. IGORR8 (Munich 2001): C.H. Lee et al. p.159

http://hanaro2005.kaeri.re.kr/data/proceeding/CN-O-17.pdf 
           by CH Lee and T. Kawai

## Mainz TRIGA

Organisation:	JOHANNES GUTENBERG Universität Mainz

Installation:	FRMZ operating since 1965

Type: 	TRIGA II reactor, 0.1 MW steady state, 250 MW peak (pulsed), 5·1014 n/cm²s pulsed

Address:	Institut für Kernchemie
Fritz-Strassmann-Weg 2, D-55128 Mainz, Germany

Home page:	http://www.kernchemie.uni-mainz.de/

Contact: 	Norbert G. Trautmann

Phone / fax:	+49 6131 39-25847  	fax:	-24488

e-mail:	norbert.trautmann@uni-mainz.de

Co-worker:	Klaus Eberhardt (klaus.eberhardt@uni-mainz.de), phone: -25846

No CNS, but

UCN:		A group around Werner Heil (wheil@mail.uni-mainz.de) and Stephan Paul (stephan.paul@physik.tu-muenchen.de) uses the TRIGA reactor (possibly in the pulsed mode) for testing successfully new methods of UCN-production (solid D2  and CD4) and UCN storage. An upgraded UCN source will be in operation from 2008 on at the radial channel D with its factor of 6 higher neutron flux.

recent publ.: 	"A solid Deuterium UCN Source at the research reactor TRIGA Mainz" by S. Paul, W. Heil, J.V. Kratz, I. Altarev, K. Eberhardt, et al. in annual report 2004 Kernchemie Uni-Mainz.

"A Prototype of the Mini-D2 Source for Ultra-Cold Neutrons" by I. Altarev, A. Frei, E. Gutsmiedl, F.J. Hartmann, et al., 2006, see bibliography on the ottosix site.

"First results with the solid deuterium UCN source at the reactor TRIGA Mainz" by W. Heil at the 6th UCN/CN Workshop in Russia 2007.

## Mianyang

Organisation:	China Academy of Engineering Physics 
Institute of Nuclear Physics and Chemistry

Installation:	CMRR under construction

Type: 	pool reactor 20 MW, Øth = 3.4·1014 n/cm²s,

Address:	P.O. Box 919-211 Mianyang, Sichuan, 621900, P.R. China

Home page:	http://www.caep.ac.cn/ ???

Contact: 	Prof. Zhao Peng Ji

Phone / fax:	+86-816-2493842		fax:	+86-816-2281598

e-mail:	zhaozhen@my-publie.s.c.cninfo.net
(general e-mail: npc@caep.ac.cn)

Co-workers:	Prof. Dai Jun-long, Eng. Hu Chun-ming (chunming@caep.ac.cn),
Prof. Shen Wen-de (phone: +86-816-2485272)

CNS:	one, operation planned for 2007

moderator: 	H2 		phase:		liquid (subcooled)

volume: 	~1 L		temperature:		18 K

pressure:	250 kPa	heat removal: 	thermal siphon (vertical)

material: chamber:	Al alloy		vacuum thimble:	Al alloy

power:	1500 W		refrigerator:		??

----------------------------------------------------------------------------------------------------

remarks:	reactor behavior in case of CNS refrigeration failure:

The reactor can continue operation with CNS emergency cooling at room temperature.

recent publications:	 "COLD NEUTRON SOURCE AT CMRR" by Hu Chunming, & al. 
(CMRR), and V. Kouzminov, & al. (PNPI) in Proc._IGORR9 2003

V. Mityukhlyaev in Proc._UCN-CNS_4th_Workshop (PNPI 2003)

## Oak Ridge HFIR

Organisation:	Oak Ridge National Laboratory (DOE)

Installation:	HFIR, operating since August 1965,

Type: 	high flux compact core reactor, HEU, 85 MW, 
Øth=15·1014 n/cm²s,

Address:	ORNL, P.O.Box 2008
Oak Ridge, TN 37831-6430, USA

Home page:	http://www.ornl.gov

Contact: 	Douglas ("Doug") L. Selby

Phone / fax:	+1 865 574 6161	fax:	+1 865 241 1887

e-mail:	selbydl@ornl.gov

CNS:	one, operation planned for 2007

moderator: 	H2			phase:		supercritical

volume: 	505 ml		temperature:		20 K

pressure:	1.5 MPa	heat removal: 	forced (cryogenic circulator)

material: chamber:	Al 6061		vacuum thimble:	Al 6061

power:	3000 W		refrigerator:		2 M$

----------------------------------------------------------------------------------------------------

remarks:	reactor does not need to shut down in case of CNS failure
(80 K (LN2) cooling loop).

recent publications: Proc. IGORR10 (NIST 2005):" HIGH FLUX ISOTOPE REACTOR
                                COLD SOURCE SAFETY ANALYSIS" by D.H. Cook

IGORR2007 (Lyon, FR, 2007): "Status of the High Flux Isotope 
                                Reactor and the Reactor Scientific Upgrades Program" 
                                by D. L. SELBY (and more, see transactions rrfm2007)

## Oak Ridge SNS

Organisation:	Oak Ridge National Laboratory + 5 partner labs (DOE)

Installation:	SNS (operational with 1.4 MW since 2006)

Type: 	spallation source, 2 MW, 1 GeV protons, Øth=15·1014 n/cm²s (peak),

Address:	project office: 701 Scarboro Road
Oak Ridge, TN 37830, USA

Home page:	http://www.sns.gov/

Contact: 	Ian Anderson (experimental facilities)

Phone / fax:	+1 865-574-0548	fax:	+1

e-mail:	andersonian@sns.gov

CNS contact:	Eric B. Iverson (iversoneb@sns.gov)

CNS:	two, operational in 2007

moderator: 	H2		phase:		supercritical

volume: 	2X1 L		temperature:		20 K

pressure:	1.5 MPa	heat removal: 	forced (cryogenic circulator)

power (total):	3500 W	refrigerator:		by AL

----------------------------------------------------------------------------------------------------

remarks:

recent publications: ICANS-XVI (Neuss 2003): M31 (A.Crabtree)

ACNS 4, June 2004: "Technical Concepts for a Long-Wavelength Target Station for the Spallation Neutron Source" by J. M. Carpenter

ICANS18- Id169, 2007: "First Moderator Performance Calculations for a SNS Second Target Station" by F.X. Gallmeier

ICANS18-Id171, 2007: "SNS Hydrogen Moderator System - A Year in Review"
by J. A. Crabtree.

----------------------------------------------------------------------------------------------------

UCN project status and activities on the Fundamental Physics Beam Line FPBL see http://www.phy.ornl.gov/nuclear/neutrons/ProjectStatus.pdf

"Status of the Fundamental Neutron Physics Beamline at the SNS" by M. Cooper at 5th UCN/CN Workshop in Peterhof 2005

ICANS-18-Id182 2007: "Fundamental Neutron Physics Beam at the Spallation Neutron Source" by W. M. Snow, & al.

## Raleigh NCSU

Organisation:	North Carolina State University

installation:	Pulstar

type: 	TRIGA reactor, light water, 1 MW, 4% LEU, 
1012 n/cm² s (< 10meV)

address:	NCSU Raleigh, NC 27695 7909, USA

home page:	http://www.ne.ncsu.edu/NRP/pulstar.html

contact: 	Prof. Bernard W. Wehring

phone / fax:	+1-919-515-4599		fax:	-513-1276

e-mail:	bwwehrin@ncsu.edu

co-workers:	Chris Gould (Chris_Gould@ncsu.edu), 
Albert Young (Albert_Young@ncsu.edu)

--------------------------------------------------------------------

planned installation in the thermal column:

Cold pre-moderator		solid CH4

UCN moderator		solid D2 < 20 K

recent publ.: 		Proc. UCN-CNS_4th Workshop (PNPI 2003) by A. Young

"A Preview of Research Projects at NC State University PULSTAR Reactor" by A. Cook at IGORR10 at NIST, Sept. 2005.

## Saclay CEA

Organisation:	CEA Saclay

Installation:	Orphée, operating since 1980, since 2003 with fewer cycles/yr,
due to budget problems.

Type: 	pool reactor, 14 MW, HEU, D2O reflector, Øth=3·1014 n/cm²s,

Address:	91191 GIF SUR YVETTE, France

Home page:	www-llb.cea.fr/

Contact: 	Mr. J. Estrade

Phone / fax:	+33 169 08-4611, fax: -8320

Co-worker:	Mr. Burdon (burdon@drnsac.cea.fr), ext. -4040

CNS:	two, refurbished in 1995

moderator: 	H2			phase:		liquid

volume: 	1 L each		temperature:		20 K

pressure:	90 kPa		heat removal:	gravity (thermal siphon)

material: chamber:	AlMg3			vacuum thimble:	AlMg3

power:	650 / 500 W		refrigerator:	AL, screw, 250 kW(el.)

----------------------------------------------------------------------------------------------------

remarks:	reactor has to stop in case of CNS failure

------------------------------

recent publications: www-llb.cea.fr/spectros/pdf/neutron-source-llb.pdf

------------------------------

Experimental facilities: Pierre Monceau at Laboratoire Léon Brillouin (LLB), 
monceau@llb.saclay.cea.fr

## Serpong

Organisation:	NATIONAL NUCLEAR ENERGY AGENCY (BATAN)

Installation:	"G.A.Siwabessy" reactor, operating since 1987

Type: 	pool reactor, 30 MW, H2O moderated, Øth=2.5·1014 n/cm²s

Address:	P2TRR-BATAN, PUSPIPTEK COMPLEX, OB NO. 31
SERPONG, TANGERANG 15310, Indonesia

Home page:	http://www.batan.go.id/

Contact: 	Hudi Hastowo (member of IGORR)

Phone / fax:	+62 21 7560-908,	 fax: -573

e-mail:	prsg@cbn.net.id

CNS:	none installed, but still planned “to be installed one day”

----------------------------------------------------------------------------------------------------

recent publications: ICANS-XV (Tsukuba 2000): status: 3.3

## Tokai-mura JRR

Organisation:	JAERI

Installation:	JRR-3M

Type: 	pool reactor, 20 MW LEU, D2O-reflector, Øth=2·1014 n/cm²s,

Address:	Jaeri, Tokai-mura, Naka-gun
Ibaraki-ken 319-1195, Japan

Home page:	http://neutrons.tokai.jaeri.go.jp/eng/

Contact: 	Dr. Yukio Morii

Phone / fax:	81-292 82 6093 fax 81-292 82 5922

e-mail:	morii@neutrons.tokai.jaeri.go.jp

CNS:	one, operating since 1990

moderator: 	H2			phase:		liquid

volume: 	1 L			temperature: 	20 K

pressure:	120 kPa		heat removal:	gravity (thermal siphon)

power:	350 W

refrigerator	TAISAN, Brayton, one screw, 320 kW(el.)

----------------------------------------------------------------------------------------------------

recent publications: 	M.Utsuro in Proc. Workshop on Cold Sources at LANSCE, 
Los Alamos, 1990, p.105

"Cold neutron production in solid and liquid CH4 moderators" by N. Morishima and T. Mitsuyasu in in Nucl.Inst&Meth. in Physics A, Volume 517,1-3, 2004, 295-300.

"Cold neutron production in solid and liquid CH4 moderators. II: on the reentrant-hole configuration" by T. Mitsuyasu, N. Morishima, & al. in Nucl.Inst&Meth. in Physics A, Volume_537,3, 2005, 610-613.

## Tokai-mura JSNS

Organisation:	JAERI

Installation:	JSNS in J-PARC

Type: 	spallation source, 3 GeV, 0.333 mA, Hg target, Be reflector

Address:	JAERI, Tokai-mura, Naka-gun
Ibaraki-ken 319-1195, Japan

Home page:	http://jkj.tokai.jaeri.go.jp/

Contact: 	Dr. Yujiro Ikeda

Phone / fax:	81-292 		fax 81-

e-mail:	ikeda@cens.tokai.jaeri.go.jp

Cold Moderators:	three, under construction, ready in 5/2008

moderator: 	H2			phase:		supercritical

volume: 	ca. 1 L ea.		temperature: 	20 K

pressure:	2 MPa			heat removal:	forced circ. ?

refrigerator power needed		5 kW

----------------------------------------------------------------------------------------------------

recent publications: Y. Ikeda in Proc. ICANSXVI (Neuss 2003) (about JSNS)

Y. Kiyanagi in Proc. UCN-CNS 4th Workshop (PNPI 2003)

------------------------------------------

## Tsukuba KEK

Organisation:	Tsukuba National Laboratory for High Energy Physics

Installation:	KENS

Type: 	spallation source, 500 MeV,

Address:	KEK, 1-1 Oho, Tsukuba-shi, Ibaraki 305-0801, Japan

Home page:	http://neutron-www.kek.jp/

Contact: 	Susumu Ikeda

Phone / fax:	+81 298 64 56 18 / +81 298 64 32 02

e-mail:	susumu.ikeda@kek.jp

Co-worker: 	Masayoshi Kawai (masayoshi.kawai@kek.jp)

CNS:			one, since 1980

moderator: 	CH4			phase:		solid

mass: 	0.378 kg		temperature:	 	27 K

pressure:	0 MPa			power:		7 W

refrigerator	Stirling 70 W		heat removal: 	directly cooled

----------------------------------------------------------------------------------------------------

Remarks:	target-moderator-reflector assembly renewed in 2001

recent publications:	ICANS-XV (Tsukuba 2000): status: 2.4, 
ICNS 2001 (Munich): poster C-9 (M. Kawai)

--------------------------

UCN group around Yasuhiro Masuda (e-mail: yasuhiro.masuda@kek.jp)

using superfluid 4He cooled below 1 K

recent publications:		ICANS-XV (Tsukuba 2000): 22.1, 22.4, 
UCN_3 Workshop 2001 in Pushkin (see Gatchina)

------------------------------------------

New spallation neutron source project with CNS: JAERI-KEK Joint Project "j-parc", home page: http://j-parc.jp/MatLife/en/index.html

## Vienna (Wien)

Organisation:	Austron

Installation:	cancelled

Type: 	spallation source (0.5 MW, 1.6 GeV, 10 Hz, 
370·1014 n/cm²s peak)

Address:	Austron c/o Atominstitut 
Stadionallee 2
1020 Wien, Austria

Home page:	http://www.austron.at/

Contact: 	Helmut Rauch

Phone / fax:	+43 1 58 801 141-00,	fax: -99

e-mail:	rauch@ati.ac.at

CNS:	studies cancelled

----------------------------------------------------------------------------------------------------

remarks:	Atominstitut Vienna runs a Triga reactor (0.25 MW) famous for its work in neutron optics.

UCN: This TRIGA reactor could be a perfect candidate for the installation of a UCN
          source

## Villigen SINQ

Organisation:	Paul Scherrer Institut (PSI)

Installation:	SINQ

Type: 	spallation source (steady state), 
Øth=0.8·1014 n/cm²s, 590 MeV,1.2 mA

Address:	Paul Scherrer Institut, CH-5232 Villigen PSI, Switzerland

Home page:	http://www.psi.ch

Contact: 	Harald Spitzer

Phone / fax:	+41 56 310-2582 	 fax:	-3131

e-mail:	harald.spitzer@psi.ch

Secretary: 	Renate Bercher (bercher@psi.ch), phone -3402

CNS:	one, operating since 1996

moderator: 	LD2 			phase:		boiling

volume: 	20 L			temperature		25 K

pressure:	150 kPa		heat removal:	thermal siphon

material: chamber:	AlMg3			vacuum thimble:	AlMg3

power:	800 W @ 1.2 mA + 165 W heat loss without proton beam

refrigerator:	Linde , 2 turbines , Sulzer reciprocating compressor
available refrigeration power    3500W

----------------------------------------------------------------------------------------------------

Remarks:	CNS refurbished by 2003 with re-entrant hole
proton beam has to stop in case of CNS failure

recent publications: ICANS-XV (Tsukuba): status: 2.6 (G.S. Bauer)
ICANS-XVI (Neuss 2003) Poster H. Spitzer

## Villigen SUNS

Organisation:	Paul Scherrer Institut (PSI)

Installation:	SUNS (at the old Piotron site)

Type: 	spallation source (pulsed) for UCN production, 
Øth=0.8·1014 n/cm²s, 590 MeV,1.2 mA

Address:	Paul Scherrer Institut, CH-5232 Villigen PSI, Switzerland

Home page:	http://ucn.web.psi.ch/

Contact: 	Manfred Daum

Phone / fax:	+41 56 310-3668	 fax:	-

e-mail:	manfred.daum@psi.ch

Secretary: 	Anita van Loon (anita.vanloon@psi.ch), phone:   -3254

co-workers:	Reinhold Henneck 	-5157, 	Klaus Kirch     -3278

UCNS:	one, under construction

moderator: 	D2 			phase:		solid

volume: 	2 L			temperature		5 K

pressure:	0 kPa			heat removal:	forced convection, He gas

material: chamber:	AlMg3			vacuum thimble:	AlMg3

power:	800 W @ 1.2 mA + 165 W heat loss without proton beam

refrigerator:	Linde / Sulzer,  available refrigeration power    3500W

----------------------------------------------------------------------------------------------------

Remarks:	project in collaboration with PNPI Gatchina

recent publications: Proc. UCN-CNS Workshop 4 (PNPI 2003) (M. Daum)

see also International Workshop on "Present Status and Future of Very Cold Neutron Applications" at Paul Scherrer Institute - Switzerland in February 2006.

## Theory and Computation Groups

(those not attached to a CNS site)

## Bariloche

CAB (CNEA) at Instituto Balseiro

Address: 	Av. Bustillo km 9500
8400 Bariloche, Argentina

Contact:	Rolando Granada (granada@cab.cnea.gov.ar)

tel. / fax:  	+54 2944 4452-23, fax -99

develops scattering kernels for cold moderator substances

recent pub.:	IGORR 7 (Bariloche 1999), 
ICANS-XV (Tsukuba 2000): 18.11.
ICANS18- Id_117 and Id_119

## Florence

CNR, Instituto di Elettronica Quantistica,

Address:	Via Panciatichi 56/30, 50127 Firenze, Italia
M. Zoppi (zoppi@ieq.fi.cnr.it) et al. study and
measure scattering kernels for cold moderators at ISIS
recent publication:	ICNS2001 (Munich): Poster C-58
			Proc. ILL Millennium Symposium (4/2001) p. 95

## Stuttgart

Universität Stuttgart,  IKE RP

Address:	Pfaffenwaldring 31
70550 Stuttgart

Contact:	Dr. Wolfgang Bernnat

tel./ fax:	+49 711 685-2118, fax:   -2010

e-mail:	bernnat@ike.uni-stuttgart.de

studies "Scattering Laws and Cross Sections for Moderators and Structure Materials for Calculation of Production and Transport of Cold and Ultra-cold Neutrons", does Monte-Carlo simulations.

recent publication:	ND2001 (International Conference on Nuclear Data for Science
                                and Technology) in Tsukuba, Japan, Oct.2001

## Organisations, Meetings

## ACoM

Organisation:	International Collaboration on Advanced Cold Moderators

goals:	investigating and developing alternative routes for employing methane and searching for similarly adequate alternative moderators.

Address:	Forschungszentrum Jülich, IFF
Postfach 19-13
52425 Jülich, Germany

home page: 		http://www.fz-juelich.de/
Contact: 		Dr. Harald Conrad 		or Dr. Günter Bauer

phone / fax:		+49 2461 61-6740, 	fax:	-2610

e-mail:		h.conrad@fz-juelich.de 	or guenter.bauer@fz-juelich.de

meetings:		e.g.: ACoM IV in 2/99 in Jülich,

ACoM V in 3/01 in Berlin,

ACoM VI in 9/02 in Jülich,

ACoM VII in 4/06 in Bariloche (AR)

## ECNS, ICNS, and ACNS

###### ECNS

The "European Conference on Neutron Scattering" (ECNS) series is organized every four years by the European Neutron Scattering Association ENSA 
(home page:  http://neutron.neutron-eu.net/n_ensa/).

The Chairman of ENSA is now Peter Allenspach, PSI.

The first of this series was held in Interlaken (CH) in 1996,

the second ECNS  in Budapest (HU) in 1999,

the third ECNS took place in Montpellier (FR) in Sept. 2003,

ECNS 4 happened in Lund (SE) in June 2007

ENSA awards every two years the prestigious Walter Hälg Prize.

###### ICNS

The "International Conference on Neutron Scattering" series was organized every three years by the local Neutron Scattering Association of the place where it takes place. From 1997 on it is held every four years in alternation with the ECNS.

There is an International Advisory Committee of about 26 members.

The last meetings:

ICNS’82 in Hakone (JP),					ICNS’85 in Sante Fe (USA),

ICNS’88 in Grenoble (FR), 				ICNS’91 in Oxford (GB),

ICNS’94 in Sendai (JP), 					ICNS’97 in Toronto (CA),

ICNS 2001 in Munich (DE), proceedings in Appl. Phys. A (Suppl.1), 2002

ICNS 2005 was be held in Sydney (AU),

ICNS 2009 will be held in Knoxville, TN (USA).

###### ACNS

The first "American Conference on Neutron Scattering" (ACNS) was organized by the Neutron Scattering Association of America (NSSA), in conjunction with the SNS-HFIR User Group (SHUG), in Knoxville, Tennessee, in June 2002.

Chairman is Rob Briber (UMD) (e-mail: rbriber@eng.umd.edu),

ACNS home page: http://www.sns.gov/acns/

ACNS2008 (American Conference on Neutron Scattering, n°4) to be held in Santa Fe, NM in Mai 2008.

## IAEA

International Atomic Energy Agency

Physics Section, NAPC

Dept. of Nuclear Sciences and Applications

P.O. Box 100

Wagramer Strasse 5

International Atomic Energy Agency

A-1400 Vienna, Austria

Contact:	Gunter Mank (g.mank@iaea.org)

Home page:	http://www.iaea.or.at/worldatom/rrdb

Data from the IAEA’s Research Reactor Database (RRDB) provides information with respect to the status of the world’s research reactors.

Each year the IAEA sends out questionnaires to the owners and operators of research reactors requesting an update on information relating to each facility. This information is fed into the IAEA’s database and comprises the Research Reactor Database (RRDB).

The research reactor web site also contains a calendar of research
reactor meetings and activities. Previous conference: 11/2003 in Santiago de Chile.
Amongst the publications: "Safety Aspects of Cold Neutron Sources" by J.M. Astruc and K.H. Gobrecht.

Next International Conference on Research Reactor Utilization: see www.ottosix.com/events.htm

- In IAEA’s "SUBPROGRAMME ON RESEARCH REACTORS:
TECHNOLOGY AND NON-PROLIFERATION" talk given at IGORR2007 in Lyon by
P. Adelfang, et al., you can read:

3.2. Project D.2.0.2: Supporting RR Modernization and Innovation

Member States, especially developing Member States, involved in planning or carrying out refurbishment and modernization of RRs often look to the IAEA for advice and assistance and to exchange information and ideas. Similarly, IAEA assistance is requested when new RRs or major  innovative systems, such as in-core loops or cold sources, are being planned or constructed. Regional and interregional thematic collaborations, networking and centres of excellence are being increasingly considered worldwide as an appropriate way to enhance utilization of RRs. This project is designed to fulfil these needs by collecting and sharing relevant information, including best practices and lessons learned.

Two important safety guides have been published earlier by IAEA:

"SAFETY GUIDE ON SAFETY ASSESSMENT OF RESEARCH REACTORS AND PREPARATION OF THE SAFETY ANALYSIS REPORT" (Safety Series 35-G1)

and "SAFETY GUIDE ON SAFETY IN THE UTILIZATION AND MODIFICATION OF RESEARCH REACTORS" (Safety Series 35-G2)

other recent publications: ICANS18_Id74
An IAEA Coordinated Research Project on "Improved production and utilization of short pulsed, cold neutron beams", presented by F._Mulhauser, G. Mank, and M. Furusaka.

## ICANS

International Collaboration on Advanced Neutron Sources

Home page: http://www.pns.anl.gov/related/icans.shtml

ICANS is an informal network of laboratories whose scientists are involved in developing pulsed neutron sources and accelerator based spallation neutron sources. The attention of the Collaboration is on the subjects of accelerator development, source (target) and moderator development, and neutron scattering instrumentation development.

Meetings:

ICANS I - Argonne National Laboratory, December 12-15, 1977

ICANS II - Rutherford Laboratory, July 10-14, 1978

ICANS III: Conference on Target Stations and Accelerator Technology. Los Alamos Scientific Laboratory, March 19-22, 1979.

ICANS IV at KEK, Tsukuba, October 20-24, 1980.

ICANS V in Jülich, June 22-26, 1981.

ICANS VI at Argonne National Laboratory, June 29-July 2, 1982

ICANS VI at Chalk River Nuclear Laboratories, September 13-16, 1983.

ICANS VIII at Keble College, Oxford, 8-12 July 1985.

ICANS-IX at PSI, Villigen, September 22-26, 1986.

ICANS-X at Los Alamos, 3-7 October, 1988.

ICANS-XI at KEK, Tsukuba, October 22-26, 1990.

ICANS-XII at the Cosener's House, Abingdon, Oxfordshire, UK, May 24-28, 1993.

ICANS-XIII at PSI, Villigen, Switzerland, October 11-14, 1995.

ICANS XIV in Utica, Illinois, USA, June 14-19, 1998.

ICANS-XV in Tsukuba, Japan, November 6-9, 2000

ICANS-XVI in Neuss, Germany, May 12-15, 2003

ICANS-XVII in Santa Fe, NM, USA, April 24-29, 2006

ICANS-XVIII in Guangdong, CN, April 23-27, 2007

## IGORR

International Group on Research Reactors

IGORR Charter:

The International Group on Research Reactors was formed to facilitate the sharing of knowledge and experience among those institutions and individuals who are actively working to design, build, and promote new research reactors or make significant upgrades to existing facilities.

Home page: http://www.igorr.com/

Actual chairman: Joel GUIDEZ by interim since June 2006

IGORR1 	in Knoxville, Tennessee 	in 3/90

IGORR2 	in Saclay 			in 5/92

IGORR3 	in Tokai, Ibaraki, 		in 9/93

IGORR4 	in Gatlinburg, TN 		in 5/95

IGORR5 	in Aix-en-Provence 		in 11/96

IGORR6 	in Taejon, Korea		in 5/98

IGORR7 	in Bariloche, Argentina 	in 10/99

IGORR8 	in Munich, Germany 	in 4/2001

IGORR9 	in Sydney, Australia 	in 3/2003

IGORR10	in Gaithersburg, USA	in 9/2005 (jointly with TRTR)

IGORR2007 	in Lyon, France	 	in 3/2007 (jointly with RRFM)

IGORR2009	in Beijing, China 		in ?/2009 (jointly with RRFM)

Proceedings of IGORR 6 to 11: see home page http://www.igorr.com/

no IGORR Newsletters any more

## People

| Altarev, Igor Garching FRM2 Bauer, Gunter ACoM Beliakov, A.A. Dubna JINR Bendiksen K.H. Kjeller Bernnat, Wolfgang Theory (Stuttgart) Bowles, Tom Los Alamos Briber, Rob ACNS Broome, Tim Chilton ISIS Burdon, Mr. Saclay CEA Carpenter, Jack Argonne Chen, Dong Feng Beijing Choi, Chang Oong Taejon HANARO Chun-ming, Hu Mianyang Conrad, Harald Jülich FZ Cywinski, Bob ECNS (ENSA) Daum, Manfred Villigen SUNS Dewey, Scott Gaithersburg Eberhardt, Klaus Mainz Estrade, J. Saclay Geltenbort, Peter Grenoble Gistau, Guy Preface Gobrecht, Klaus HYERES, France Gould, Chris Raleigh NCSU Granada, Rolando Theory (Bariloche) Greene, Jeff Gaithersburg Grosz, Tamas Budapest Gutsmiedl, Erwin Garching Guyon, Hervé Grenoble Hargitai, Tibor IAEA Hartmann, Joachim Garching Hastowo, Hudi Serpong Heil, Werner Mainz Henneck, Reinhold Villigen SUNS Hoffmann, Diane Argonne Huffman, P.R. Gaithersburg Huang, Yu-Hao Lungtan Hunter, Brett Lucas Heights Ikeda, Susumu Tsukuba Ikeda, Yujiro Tokai-mura JSNS Iverson, Eric Oak Ridge Jun-long, Dai Mianyang Kawabata, Yuji Kyoto Kawai, Takeshi Kyoto Kawai, Masayoshi Tsukuba Kirch, Klaus Villigen SUNS Knop, Wolfgang Geesthacht Knudsen, Kenneth D. Kjeller Konoplev, Kir Gatchina Krohn, Herbert Berlin | Kiyanagi, Yoshiaki Tokai-mura JSNS Lee, Albert G. Chalk River Lee, C.H. Lungtan Lee, Kye - Hong Daejeon Leuschner, Mark Raleigh NCSU Luzheng, Yuan Beijing Miller, Ross Lucas Heights Mityukhlyaev, Victor Gatchina PNPI Morii, Yukio Tokai-mura Morishima, N. Kyoto RRI Nesvizhevsky, V. Grenoble ILL O'Kelly, Donna J. Austin, Texas Paranjpe, Shriniwas IAEA Paul, Stephan Garching FRM2 Peng Ji, Zhao Mianyang Rauch, Helmut Vienna (Wien) Robinson, Rob Lucas Heights Rosta, Laszlo Budapest KFKI Russell, Gary Los Alamos Saunders, Alexander Los Alamos Schott, Wolfgang Garching FRM2 Schrie, Niki Chalk River Seehoye, Denis Chalk River Selby, Doug Oak Ridge HFIR Serebrov, Anatoli Gatchina PNPI Shabalin, Evgeni Dubna JINR Shen, Feng Beijing Shokr, Mohammed Cairo Spitzer, Harald Villigen SINQ Stelzer, Hermann Jülich FZ Svensson, Eric Chalk River Swainson, Ian P. Chalk River Taylor, A.D. Chilton ISIS Thomas, Antje Berlin Tözsér, Sándor BudapestKFKI Trautmann, N.G. Mainz Triga Utsuro, Masahiko Kyoto RRI van Loon, Anita Villigen SUNS Verkooijen, Adrian Delft HOR Wehring, Bernard Raleigh NCSU Wen-de, Shen Mianyang Williams, Bob Gaithersburg NIST Wolters, Johannes Jülich FZ Yang, J.H. Beijing Yang, Tsun-Neng Lungtan Young, Albert Raleigh NCSU Zakharov, Arcady Gatchina PNPI Zoppi, M. Theory (Florence) |
| --- | --- |

## Glossary

for the World Directory of Cold and Ultra-cold Neutron Sources

| Cold Neutron Source (CNS) | a facility producing cold neutron beams from the thermal neutrons generated by a neutron source. |
| --- | --- |
| Cold Neutrons | free neutrons in the energy range below 20 meV |
| Cold Neutron Wave Length | from 0.2 nm (energy 20 meV) to more than 2 nm (0.2 meV) |
| Cold Neutron Velocity | from 2000 m/s (energy 20 meV) to less than 200 m/s (0.2 meV) |
| Cold Neutron Temperature | from 220 K (energy 20 meV) to less than 3 K (0.2 meV) |
| Cold Neutron Moderator | a liquid or solid which is cooled to below 100 K in order to shift the spectrum of incoming thermal neutrons to lower energies. A good moderator has a high scattering cross section combined with low absorption cross section (examples: liquid hydrogen, solid methane). |
| Cold Neutron Gain | a measure of the performance of the CNS, in fact the ratio of the neutron flux given by the operating CNS to the flux if the CNS were not there. This ratio is wavelength dependant. It can reach values well above 100 at very long wavelengths. |
| Heat Removal | a lot of heat is deposited by nuclear radiation in the cold neutron moderator, the latter being usually close to the radioactive neutron source. Powerful refrigeration at very low temperatures and efficient heat transport from the moderator to the refrigerator are needed, see thermal siphon. |
| Neutron Source | a neutron generator like the core of a nuclear fission reactor (mostly giving a steady state neutron output), or the target of a high energy proton accelerator (mostly for pulsed neutron beams), see Spallation Source. |
| Neutron Guide | an evacuated channel, up to 100 m long, containing and confining the neutron beam. The colder the neutrons the better they are transported in such a guide. |
| Spallation Source | a dedicated target station at a high power proton accelerator for neutron production by spallation of heavy nuclei like Zr, W, Pb, Hg, or U. Cold neutron beams are obtained by placing a cold moderator in the vicinity of the target. |
| Thermal Siphon | the heat developed in the cold moderator by nuclear radiation can be removed either by direct contact with the refrigerator gas (usually He) or by convection in the moderator fluid, the latter extending from the active moderator volume to a heat exchanger. The convection can be forced (circulator) or natural. The natural convection loop is called a thermal siphon. |
| Ultra-cold Neutrons (UCN) | neutrons with energies below about 1 meV. They totally reflect at certain walls, i.e. one can keep them in a bottle until they decay. |
| Ultra-cold Neutron Source | although UCN are present in CNS (at a very low density), dedicated UCN Sources are nowadays developed, the cold moderator being e.g. solid deuterium below 10 K or superfluid helium below 1 K. |